Robotic technologies for on-site building construction: A systematic review

M.M. Gharbiaa, A.Y. Chang-Richardsa and R.Y. Zhongb

aDepartment of Civil and Environmental Engineering, University of Auckland, New Zealand

bDepartment of Industrial and Manufacturing Systems Engineering, University of Hong Kong, Hong Kong

E-mail: mgha253@aucklanduni.ac.nz, yan.chang@auckland.ac.nz, zhongzry@hku.hk

Abstract

Robotic technologies for on-site building construction represent a major departure from conventional construction approaches. The new paradigm brings a host of new topics into the forefront of robotics in construction research. To gain an improved understanding of the trend and trajectory of research on application of robotic technologies in construction, this paper presents a systematic review on 86 articles. Cross-comparison analysis suggests that five categories of topic have become major themes in this research domain, namely, types of construction robots, implementation in site, innovation in material and design, building elements and construction projects, and robotics features. The review suggests that traditional automated systems have been in use to streamline construction process, while construction moves into an era of technology innovation, such as using 3D printing and modular construction, there is a great need for more robotics research and development (R&D) for achieving efficiency gains in a construction process.

Keywords: Robotics, building construction, automation, systematic review

Introduction and background

Off-site fabrication in building construction has grown rapidly since the construction industry began to embrace the robotic technologies in controlled off-site environments [1]. According to the Modular Building Institute in USA [2], prefabrication offers the potential to increase construction efficiency, enable better sequencing in the construction process and reduce weather-related holdups. It also offers benefit of reducing a project’s delivery time and construction cost and creating a safer working environment [2]. However, prefabrication technology has been facing some issues that have had hampered its widespread usage, including misconception about quality and lack of ability and flexibility to customise, limited potential to be used in high-rise buildings, and in some cases, the high cost of transportation [3].

The productivity gains by using industrial robots over the past decades have helped open up new application areas of robotics in building construction [4]. As construction activities become more complex and challenging for large and mega-infrastructure and commercial building projects, robotic technologies may offer opportunities for implementing new building construction methods to deal with these complexities [1],[5]. In most cases, buildings tend to be individually unique and prototypical, but the construction process in terms of its organizational structure, project management method, and construction activities can be replicable, which is amenable to automation principles [6].

The construction industry provides a rich context for the investigation of many research questions in relation to the development and application of robots in a variety of ﬁelds [7]. What types of on-site construction activities can be undertaken by automated robots to replace manpower or skilled worker? To answer this question, the first step is to investigate the current status of research on robotics in construction. For this purpose, this paper applies a systematic review to evaluate and synthesize empirical findings on the use of robotic technologies in on-site building construction. As the construction industry is upgrading and transforming in the era of Industry 4.0, a state-of-the-art review will provide insights into existing gaps and areas for technological improvement. It also presents the digital industry with an improved understanding of the application areas of different types of robotic technology in construction and potential technical domains that require further investment in research and development.

The remainder of the article is structured as follows. Section 2 introduces the systematic review methodology. Section 3 and 4 detail the results and discussion. Finally, the paper concludes the findings of this review and reflects on the future research directions that can advance the use of robotics in improving construction practice.

Methodology

To provide a robust understanding on robotic technologies in on-site building construction, a systematic review approach was adopted. In comparison with a conventional literature review, a systematic review applies an explicit, rigorous, reproducible, and auditable methodology for evaluating and interpreting all available research relating to a particular research question, topic area, or phenomenon of interest [8]. According to Cooper et al. (1986), a systematic can also overcome the shortcomings of a single facet approach which is often adopted in a literature review, by representing the bigger picture by combining discrete pieces and synthetizing results in an organized way [9]. Additional benefits also include that researchers can summarize existing evidence about a phenomenon, identify gaps in current research, and provide grounds to position or support new ideas and hypotheses [10].

The review has been undertaken in five distinct stages, including 1) development of review protocol, 2) identification of inclusion and exclusion criteria, 3) searching for relevant papers, 4) applying critical appraisal, 5) data extraction, and synthesis.

Research question

The aim of the systematic review is to locate relevant existing studies based on the research question ‘How robotic technologies have been adopted and introduced for the on-site building construction industry?’

Protocol development

The protocol for the systematic review was developed in accordance with the Preferred Reporting Items for Systematic Reviews and Meta-Analyses (PRISMA) guidelines [11]. This protocol specified the research questions, search strategy, inclusion, exclusion and quality criteria, data extraction, and methods of synthesis.

Inclusion and exclusion criteria

Papers were excluded if their focus, or main focus, was not related to robotics in construction or if they did not present empirical data. Furthermore, the research question is concerned with on-site building, therefore, papers that focused on off-site building construction were excluded. Papers were eligible for inclusion in the review if they presented empirical data on robotics in on-site building construction and passed the minimum quality threshold (see Section 2.5). The systematic review included research articles published from 1994 to and including 2018. Papers only written in English, clearly describe its methodology, completed and concluded were included.

Data sources and search strategy

The search strategy included electronic databases of Scopus, Web of Science, IEEE, and Engineering Village.  Figure 1 shows the systematic review process and the number of papers identified at each stage.

Figure 1. Flowchart of systematic review process. (PRISMA flow diagram [11])

In the identification stage, the search term (robot* AND construction) were used for searching such a term in the titles, abstracts, and keywords of the articles in the included electronic databases. These keywords are widely-known for their use in research articles. As shown in Figure 1, this search strategy resulted in a total of 65,608 documents. Of total number of documents (n = 64,506), 1,020 were book sourced, 13,696 journals, and 49,790 were resulted from a conference proceeding source.

At screening stage, duplicated items were removed as well as papers from undefined or trade publications resources. At this stage, 102 articles were excluded after removing duplicates as well as papers from undefined or trade publications resources. However, it was not always obvious whether a study was, indeed, an empirical one. Therefore, all articles that indicated some relations with robotics in construction were included. At eligibility stage, papers were excluded if their main focus was not robotics in on-site building construction. As a result, a total of 86 primary articles were included for the detailed quality assessment.

Quality assessment

The methodological quality of the eligible selected papers was critically appraised using a set of screening questions adopted from the Critical Appraisal Skills Programme (CASP) [12]. The questions used to assess the quality of these articles are presented in Table 1. The tool provides a guide for appraising qualitative research to consider if the results of the study are valid, what the results are, and the benefits of the results [12]. In this study, these questions provided a measure of the extent to which we could be confident that a particular study’s findings could make a valuable contribution to the review. Each of the 9 questions was graded on scale of (yes = 1, no = 0), and only question 1 was used as the basis for including or excluding a study.

Table 1. Quality appraisal questions

| Screening Questions |  |
| --- | --- |
| Q1 | Research: Is the paper based on research |
| Q2 | Aim: Was the aim of the research clear? |
| Q3 | Method: Was the research methodology used appropriate? |
| Q4 | Design: Did the study design address the aims of the research? |
| Q5 | Data analysis: Was the data analysis sufficiently rigorous? |
| Q6 | Findings: Are the findings clearly stated? |
| Q7 | Gaps: Have gaps in the literature been clearly identified? |
| Q8 | Acceptance: Can I accept these findings as true? |
| Q9 | Value: Can I apply these findings to my own work? |

The results of quality assessment are shown in Figure 2. Because only research papers were included in this review, all included papers were rated as yes on the first screening question, in addition, they all had a clear statement of the aims of the research. While the number of negative answers was eight for each criterion of research methodology, study design, and data analysis. Moreover, the highest numbers of negative answers were 32, and 37 as it has been noticed that the findings were not well described, and gaps in the literature were often not identified.

Figure 2. Quality appraisal summary results (out of total number of articles: 86)

Peer assessment

The titles of all articles that resulted from identification stage were scanned to determine their relevance to the systematic review. At the eligibility stage, the abstracts were divided among the first two authors and the third author in such a way that each abstract was reviewed by two researchers independently of each other. All disagreements were resolved through discussions that among all three researchers, before proceeding to the final stage. Each of the 86 articles that remained was assessed independently by the authors, according to quality assessment procedure.

Data extraction

During this stage, data was extracted from each of the 86 papers included in this systematic review according to a predefined extraction form (see Figure 3). This form enabled to record full details of the articles and be specific about how each of them addressed the research question. Data about each article, including its research aims, settings, type of research methods used, findings, and conclusions were tabulated in Microsoft Excel.

Figure 3. Data extraction and data analysis

Data analysis

Meta-ethnographic methods were used to synthesize the data extracted from the primary papers [13]. In a meta-ethnographic synthesis, articles can relate to one another in one of three ways: they may be directly comparable as reciprocal translations; they may stand in opposition to one another as refutational translations; or taken together they may represent a line of argument [14]. Table 2 shows the seven-step process for conducting a meta-ethnography. This has identified a set of interpretations and themes, which recurred across papers, through documenting the different feature for construction robots, highlighting the gaps.

Table 2. Seven phases of meta-ethnography (Noblit and Hare [13])

1. Getting started

2. Deciding what is relevant to the initial interest

3. Reading the papers

4. Determining how the papers are related

5. Translating the papers into one another

6. Synthesizing translations

7. Expressing the synthesis

Results

The detailed information about the year of publication, country of first author, article source, construction robot type, robotic system design and building category is listed in Appendix 1. We categorized the results into two main themes: 1) summary of key publications and 2) results from thematic analysis. Below we report the results in the following subsections: publishing framework and application areas, research focus, types of on-site building construction robots, implementation on a construction site, innovation in construction material and engineering design, targeted building elements and construction projects, and construction robotic features.

Summary of key publications

With respect to the types of on-site buildings that have been reviewed, Table 3 shows that 47 (54.5%) of the literature in this review were related to concrete structures. Papers about external and internal finishes came next with 27 (31.5%) of the literature. However, steel structures could be found only in 12 (14%) of the total 86 papers.

Table 3. Distribution of articles based on construction category

| Construction category | Number | Percentage |
| --- | --- | --- |
| Concrete structures | 47 | 54.5% |
| Finishes - external/ internal | 27 | 31.5% |
| Steel structures | 12 | 14.0% |
| Total | 86 | 100% |

Table 4. Distribution of articles after publication channel and occurrence

| Source | Publication channel | Number | Percent |
| --- | --- | --- | --- |
| Book | Others | 3 | 3% |
| Book series Total |  | 3 | 3% |
| Conference | ISARC | 16 | 19% |
|  | IEEE | 14 | 16% |
|  | Others | 12 | 14% |
|  | ICCAS | 4 | 5% |
|  | ARC | 2 | 2% |
|  | Automation in Construction | 2 | 2% |
|  | ACADIA | 1 | 1% |
|  | ASME | 1 | 1% |
|  | ARCM | 1 | 1% |
|  | ICARA | 1 | 1% |
|  | ICSMA | 1 | 1% |
|  | IOP | 1 | 1% |
|  | IROS | 1 | 1% |
|  | MMS | 1 | 1% |
| Conference Total |  | 58 | 67% |
| Journal | Automation in Construction | 11 | 13% |
|  | Others | 9 | 10% |
|  | Autonomous Robots | 3 | 3% |
|  | Architectural Design | 1 | 1% |
|  | IEEE | 1 | 1% |
| Journal Total |  | 25 | 29% |
| Grand Total |  | 86 | 100% |

Table 4 gives an overview of the literature according to publication channel, which indicated that conference proceedings of ISARC and IEEE in addition to Automation in Construction journal have the largest number of articles, in which they accounted for almost half of the total considered papers of 86. In general, 67% of the papers (58 out of 86) were published in conferences, while 25 (29%) appeared in scientific journals.

Regarding the year of publication, a steady increase of the papers has been noticed during the period from 2015 to 2018, in addition to 2005 to 2009. Although this could indicate for a growing interest in performing research related to the subject, the analysis results demonstrated a substantial decrease in the number of literatures from 2010 to 2014. The reason for such decline can be related to that the number of funded articles has not increased in between the periods of 2005-2009 and 2015-2018 (See Figure 4).

Figure 4. Chronological distribution of articles (total number of articles: 86)

Table 5. Publishing framework and application areas (total number of articles: 86)

| Building elements | Construction activities to be replaced | Country | Number |
| --- | --- | --- | --- |
| Non-structural | Masonry work | Germany | 5 |
|  |  | Spain | 2 |
|  |  | Portugal | 1 |
|  |  | Hong Kong | 1 |
|  |  | Japan | 1 |
|  |  | Switzerland | 1 |
|  |  | USA | 1 |
|  |  | Czech Republic | 1 |
|  |  | Serbia | 1 |
|  | Curtain-wall | South Korea | 8 |
|  |  | Netherlands | 1 |
|  | Ceiling | Spain | 1 |
|  |  | South Korea | 1 |
|  | Tiles Laying | South Korea | 3 |
|  |  | USA | 1 |
| Structural | Structural Concrete | USA | 16 |
|  |  | Germany | 6 |
|  |  | Switzerland | 4 |
|  |  | France | 4 |
|  |  | Singapore | 3 |
|  |  | UK | 3 |
|  |  | Poland | 2 |
|  |  | Spain | 2 |
|  |  | South Korea | 2 |
|  |  | Italy | 1 |
|  |  | Australia | 1 |
|  |  | Netherlands | 1 |
|  |  | Russian Federation | 1 |
|  |  | Portugal | 1 |
|  | Structural Steel | South Korea | 7 |
|  |  | Japan | 2 |
|  |  | USA | 2 |
|  |  | Germany | 1 |
|  |  | Spain | 1 |

Publishing framework and application areas

Table 5 illustrates the number of publications according to the country of origin for the first authors, in respect to the building element category and construction activities. When comparing the geographical distribution of the papers related to robotic research for non-structural building components, Germany is ahead of all others in masonry work, and South Korea for research on curtain wall. Concerning the building structural elements USA has been leading the topic of structural concrete followed by Germany and Switzerland, while authors from South Korea targeted the structural steel elements and have published more than 50% of related topics.

### Research focus

Figure 5 illustrates the distribution regarding the research focus of the analysed papers. The largest proportion of the articles in all building construction disciplines addressed the purpose of using robotic technology to improve overall construction efficiency (34 articles found in concrete structures, 26 in finishes, and 9 in steel structures). A considerably lower proportion of papers focused on introducing a new construction technology only (3 articles in concrete structures, and 3 in finishes). Followed by structural concrete articles which addressed interest to adapt the technology for Construction in extraterrestrial environments (5 articles). The minority of papers picked up reducing the high accident rate, and the construction in hazardous environment or disaster areas.

Figure 5. Research focus of papers (total number of articles: 86)

Results from thematic analysis

### Types of on-site building construction robots

As presented in Table 6, 3D printer was the most popular robotic technology among the others (22 articles), followed by automated building system (16 articles). Glazing robots, brickwork robots, swarm robotics, and wire robots have been less popular (8-10 articles each). However, technologies related to climbing robots and mobile robots were the least proposed among the reviewed papers.

Table 6. Types of on-site construction robots (total number of articles: 86)

| Type of on-site construction robots | Number | Percent |
| --- | --- | --- |
| 3D Printers | 22 | 26% |
| Automated Building System | 16 | 19% |
| Glazing Robot | 10 | 12% |
| brickwork robot | 8 | 9% |
| swarm robotics | 8 | 9% |
| Wire robots | 8 | 9% |
| Climbing Robot | 2 | 2% |
| Mobile Robotics | 2 | 2% |
| Robotics for Moulds | 2 | 2% |
| Tile Robots | 2 | 2% |
| Humanoid robot | 2 | 2% |
| Drilling Robots | 1 | 1% |
| Robotic Construction Crew | 1 | 1% |
| Robotics for Insulated Walls | 1 | 1% |
| Spraying Robotics | 1 | 1% |
| Total | 86 | 100% |

Figure 6 illustrates the distribution of articles on construction robots according to their usage in building construction. 3D printers, swarm robotics, and automated building system were the most studied robotic technologies to be used for concrete structure. Automated building construction has been the favourable technology in constructing steel structures. In contrast, glazing robots were used for building curtain wall while brickwork robot and wire robots were mainly used for masonry work. Meanwhile, internal building finishes were the area that is less likely to utilise on-site construction robots.

Figure 6. Distribution of the construction robots according to their usage in building construction (total number of articles: 86)

### Implementation on a construction site

In concerning level of development of robotic technology cited in the reviewed articles, as shown in Table 7, our analysis shows that 49 articles (57%) cited existing or developed advanced technologies for on-site building construction, while others envisaged the potential of future technologies to address certain challenges.

Table 7. Level of development

| Level of Development | Number | Percentage |
| --- | --- | --- |
| Implemented | 7 | 8% |
| Achievable but Challenging | 7 | 8% |
| Achievable | 23 | 27% |
| Challenging | 49 | 57% |
| Total | 86 | 100% |

Figure 7 summarises findings about the level of development of robotic technologies used among different building construction disciplines. Most of the mature technologies related to building finishes and steel structures. A large number of researchers found inventing a new robot for any structural concrete work to be challenging.

Figure 7. Level of development distribution among the different building construction disciplines (total number of articles: 86)

### Innovation in construction material and engineering design

In relation to innovation in construction materials, Table 8 shows that the literature can be divided into two types of studies, one introducing an alternative material (42 article) and the other focusing on standard building material (44 articles). Studies related to building finishes and steel structures utilized the conventional construction material, while 98% from the introduced alternative material could be found in concrete structures.

Table 8. Innovation in construction material

| Building discipline | Innovation in material | Number | Percentage |
| --- | --- | --- | --- |
| Standard Building Material | Structural Steel | 11 | 25% |
|  | Structural Concrete | 6 | 14% |
|  | Finishes | 27 | 61% |
| Total standard material |  | 44 | 100% |
| Alternative material | Structural steel | 1 | 2% |
|  | Structural Concrete | 41 | 98% |
| Total alternative material |  | 42 | 100% |

Despite concrete structures being the most researched building discipline, researchers in this field were still unable to introduce thorough novel construction material. In regard to alternative materials in concrete type of structures, Figure 8 shows that 45% of the papers considered mesh wire for reinforcement and around 79% from the total literature proposed polymer based material and cementitious material to replace the ordinary cement. While mortar mix, intelligent concrete blocks, and ultra-high-performance concrete found to be novel alternatives to conventional concrete mix.

Figure 8. Alternative material in concrete structures

Table 9. Innovation in engineering design

| Design innovation | Building discipline | Number | Percentage |
| --- | --- | --- | --- |
| Conventional Design | Finishes | 25 | 61% |
|  | Structural Steel | 9 | 22% |
|  | Structural Concrete | 7 | 17% |
| Total conventional design |  | 41 | 100% |
| Limited Novel design | Structural Concrete | 26 | 84% |
|  | Structural Steel | 3 | 10% |
|  | Finishes | 2 | 6% |
| Total limited novel design |  | 31 | 100% |
| Novel design | Structural Concrete | 14 | 100% |
| Total novel design |  | 14 | 100% |

As illustrated in

Table 9, the level of innovation in design among the total number of 86 papers varied from conventional to novel. 41 papers were found to still use the conventional engineering design, especially in building finishes and steel structures. Most of the articles related to concrete structures could introduce limited towards fully novel structural design (40 papers). 31 out of 86 articles have proposed the use of robotic technologies based on a novel engineering design.

### Targeted building elements and construction projects

An analysis was made on what building elements could be built on-site by the presented robotic technologies. As illustrated in Table 10, the types of building elements have been divided into two categories of: one can be built by the robot and cannot be built. The results demonstrated that building elements related to finishes and steel structures can be built on-site. While in concrete structures only vertical RC elements can be built by the proposed robotics, though, horizontal RC elements could not be built on-site unless they were supported or assembled.

Table 10. Targeted building elements

| Category | Building Discipline | Building Elements |
| --- | --- | --- |
| Can be built | Finishes - External | Masonry Walls |
|  |  | Curtain Wall |
|  | Finishes - Internal | Floor tiling |
|  |  | Plaster boards |
|  |  | Wall and ceiling Tiling |
|  |  | Ceiling Glass |
|  | Structural Concrete | Vertical RC elements |
|  |  | Most of the RC elements by assembly |
|  | Structural Steel | Complete steel structure |
| Cannot be built | Structural Concrete | Horizontal RC elements |

Table 11 gives an overview of the papers according to the types of building projects cited in the articles. 58 articles were related to low rise buildings and low to medium rise buildings, in which structural concrete masonry work and tile placing were the main topics. High rise building in addition to medium to high rise building (23 articles) could attract research attention in relation to structural steel construction and curtain-wall fixing.

Table 11. Targeted building projects

| Project | Building element | Number | Percent |
| --- | --- | --- | --- |
| High Rise Buildings | Structural steel | 10 | 50% |
|  | Curtain wall | 9 | 45% |
|  | Ceiling | 1 | 5% |
|  |  | 20 | 100% |
| Medium to High Rise Buildings | Structural steel | 2 | 67% |
|  | Structural concrete | 1 | 33% |
|  |  | 3 | 100% |
| Low to High Rise Buildings | Masonry Work | 6 | 75% |
|  | Structural concrete | 1 | 13% |
|  | Ceiling | 1 | 13% |
|  |  | 8 | 100% |
| Low to Medium Rise Buildings | Structural concrete | 16 | 67% |
|  | Masonry Work | 7 | 29% |
|  | Tile placing | 1 | 4% |
|  |  | 24 | 100% |
| Low Rise Buildings | Structural concrete | 27 | 87% |
|  | Tile placing | 3 | 10% |
|  | Masonry Work | 1 | 3% |
|  |  | 31 | 100% |

### Construction robotic features

Different types of robotic systems have been reported in the literature. Rapid prototyping and self-assembly were found to be the most appropriate systems for the purpose of concrete building construction, while controlled assembly system appeared mostly in steel structures and building finishing. Furthermore, the Cartesian motions programme attracted researchers for building finishes and concrete structures. As presented in Figure 9, the analysis shows that robotic systems vary depending on their usage in building construction.

Figure 9. Robotic system design/ programme

In terms of the types of robots in construction, 64% of articles that related to concrete structures used manipulator robots, and 17% adopted collective construction robot. While 75% of articles related to steel structures favoured assembly by multiple robots, only 17% utilised collective construction robot. In building finishes, assembly by manipulator and manipulator robot were found to be the most useful robotic categories for internal and external finishes work. As presented in Figure 10, the results are in consonance with rapid prototyping and controlled assembly systems, in addition they highlighted the most appropriate robotic categories for the purpose of on-site building construction.

Figure 10. Categories of construction robots

Discussion

Technology evolution over time and across applications

Figure 11 shows how the on-site robotic technologies have evolved over time and compares between different technologies in regard to the building types in which they were used.

It shows that early attempts in developing a mechanized construction system for concrete and steel elements led to pioneered automation system in construction. The key concept of such a system is “working floor” in which robotic systems form as a systematic factory that could be either fixed or movable [15]. A fixed factory automatically assembles one entire floor at a time and the floor is jacked up one level when it is finished, whereas a movable factory moves itself along as it completes portions of the building [15]. However, the systems highly depended on labours to weld and bolt steel members and were mainly based on conventional construction material and building methodology [16][17]. In addition, their implementation has been prohibitively expensive due to the extraordinary complexity of automating a large number of various operations such as large and heavy steel and concrete members [18].

Only until early 1990s, robotic arm in mobile platform has been developed for non-structural building components such as masonry, tiles, and curtain walls based on articulated robots placed over mobile platforms and operated in a human-robot co-operated manner [19]. However, as most robotic arms were relatively large requiring a big space to transport and place on site, its usage remains limited [20].

The recent advancements during the past 20 years in the field of industrial robots have brought hope for using novel robotic design systems such as rapid prototyping in self-organized

Figure 11. On-site robotic technologies evolution

construction and assembly. In particular, 3D-printing and digital fabrication technologies are receiving an increasing attention in the construction of structural concrete elements, due to their ability to build any shape and benefits of eliminating the need of formwork [21].

Looking into future, the self-organized assembly and construction systems such as swarm robotics and climbing robots may play a larger role as they use simple, identical, autonomous robots to assemble two-dimensional structures [22]. Comparison across the studies reviewed also shows that it looks like cable-driven robots have received a lot of attention among all other technologies in the field of robotizing the masonry work. The reason is perhaps due to the fact that their construction is relatively simple and inexpensive, and they are easy to transport and assemble [23]. Humanoid robot also attracted attention from many researchers as highly customizable technology for internal finishes. Using such technologies in real construction sites is still not possible, however, the presented prototype is showing that there is a strong promise for their future applications on sites (see Figure 11).

Figure 12. Integrated diagram: how the robotic technologies have been used in on-site building construction industry

How robotic technologies have been implemented for the on-site building construction industry?

Overview of adopted technologies

Figure 12 shows the results to the research question of how robotic technologies have been adopted and introduced for the on-site building construction industry. The analysis indicates that the potential use of robotic technology in structural building elements related to substructures and horizontal members were not fully explored. Application of robotic technology in non-structural building elements seemed to be limited to masonry, plaster boards, tiling, and curtain walls. Our analysis revealed that there is little study that offer a comprehensive technology for on-site building. The result further supports the argument of [16][24][29] that on-site building robots are currently more suitable for supporting workers in construction rather than completely replacing them.

Types of building elements and projects

The distribution of articles based on the construction category indicated that most studies seemed to be focused on robotizing the construction of concrete structures. This is in agreement with previous studies from [30][35] showed that the current robotic research focuses on constructing of concrete structures as it tended to be labour intensive. Researchers from the USA, and South Korea appear to be the pioneers to lead the research related to robotics in concrete and steel structures. According to [36][37] that as the concrete building sector in the USA is suffering from an alarming labour deficiency and the safety requirements are becoming stricter, robotized concrete work might provide a solution to addressing this issue,

The analysis on what building elements and types of projects cited in the articles, showed that most robotic technologies were aimed at multiple building projects, while only limited to certain building elements. This could highlight the need to explore the possibility of applying robotics technology in a certain type of building projects, with the intent to construct a broad range of building elements. The results are also in line with study from [6], which showed that most of the construction robotic research has been focused on using robots for construction and very few touched on its application in the design for construction robotization.

Degree of innovation in design and material

The degree of innovation in construction materials and engineering design showed that little research was conducted on novel materials and design for on-site building construction. The results in a way support that on-site construction robots are more likely to innovate through further developments in construction design and materials in a new era of architecture [38]. This also suggests that the use of innovative building material in the engineering design and 3D on-site printing solutions may revolutionize the conventional construction process and open up opportunities for innovation in construction.

Challenges and opportunities

The distribution of articles according to their publication channels and occurrence warrants a need for more coverage of this topic in journals. This is in line with the work of [5][33][35][39][40] stated that this field of study is still in its infancy and thus characterized by a limited number of papers and a relative absence of informed research. Our analysis of the level of robotic technology development further shows that although it appears to be an emerging research field, it holds the promise for applications in many types of building activities and across various building types. The chronological distribution of articles suggested a seemingly linear relationship between the level of funding support and the number of publications.  For robotic technology to reach its full potential in future construction, financial support for current and emerging research and development is critical

In addition, most of the articles (90%) reported that it is challenging to apply robotic technologies for on-site building construction due to such factors as: absence in design for automation, lack of research, inability to think out of the box and hence stifled innovation, high cost, difficulty in human-robot cooperative, and compliance with the building regulation. With the efficiency drivers to adopt more use of robotic technologies, the need to remove the barriers to innovation such as legal impediments in building code and regulations, health and safety concerns for human-robot co-operation become more pressing. This is particularly the case for modular type of construction where robotic technology can be widely applied while its quality assurance and occupational prototypes were not yet considered in building regulations of many countries.

On the other hand, all articles acknowledged that robotic technologies would offer such benefits as introducing greater efficiency into construction processes, reducing dangerous site conditions, reducing construction cost and addressing labour shortages. In particular, the efficiency gain for on-site construction activities can be enhanced by decreasing the number of low-skilled labours [41]. A recurring concern in articles has been the impact of robotics on the overall construction sector as a whole, namely what would be the implications for the construction workforce training and development, the economy and employment, and polytechnic and tertiary education. The enabling conditions including such as a legal environment, occupational practice guidelines, training and education curricula, building consenting process, health and safety regulations, are needed to be in place before a full assessment of the benefits of using robotic technology in construction can be undertaken. In this sense, there is still some way to go before the construction of unique, one-off, and highly complex structures can be made possible by innovative robots.

Conclusions

The research provides a systematic review on robotic technologies for on-site building construction, covering 86 studies found to be of acceptable rigour, credibility, and relevance. A number of reported benefits and limitations of using on-site robotics for building construction were identified.

Analysis of contents of these 86-article revealed that most of the articles focused on finding a way to robotize the production of concrete structures on site. However, as robotics in construction is an evolving research field with increased interest shown in conference proceedings especially in past several years, there is a need for more in depth investigations to be published in journals. A linear relationship between funding and the number of publications was found, which might explain the low research activities. However, researchers in Germany seemed to play a lead role in researching topics related to masonry work, while those from South Korea dominated the research on application of robotic technology for curtain wall and steel structures. Meanwhile, topics related to concrete structures have been mostly published by the USA scholars, followed by those in Germany and Switzerland. The analysis also pointed out that the largest proportion of the studies in all building construction disciplines addressed the aspect related to improve the overall construction efficiency.

The robotic technology for on-site building construction was found to be a growing application field, where 3D printer and automated building construction seemed to offer more potential to influence the development of robotics research for improved on-site building construction. It is also interesting that despite concrete structures have been the most common application area for robotic technology, the technology itself has been the most challenging problem to address. Regarding the robotic features, proposed robotic systems in literature vary depending on the specific building discipline. Overall, literature showed that rapid prototyping and controlled assembly were the most appropriate robotics categories for the purpose of on-site building construction.

The key alley for future research is related to the potential to innovate within an enabling environment. While the development of swarm technology seems to signal the possibility for new inventions in the future, there is a need to look at how various tools, professionals and robots can work together on site. The future of construction is most likely to be an integrated process that allows to take advantage of both conventional and robotic technologies at the same time. Revolutionized innovations in building construction can only be achieved with signiﬁcant progress on research in construction material, robotics hardware innovation and integration between design and construction, such in the case of the recent attempts in 3D printing. In addition, enabling conditions including such as a legal environment, occupational practice guidelines, training and education curricula, building consenting process, health and safety regulations, are needed to be in place before a full assessment of the benefits of using robotic technology in construction can be undertaken.

The results reported in this article draw upon a systematic review. The main limitation of this research methodology might be related to the probability of bias in selection of publications and inaccuracy in data extraction. The keywords for article research were discipline specific. Therefore, there is a risk that some relevant studies might have been omitted during the search. Furthermore, the adopted PRISMA protocol required the use of the predefined inclusion and exclusion criteria for selecting papers, which imply that relevant papers that do not fall into these criteria may have been neglected.

As on-site building construction is complex and currently available   automated   systems   are   geared toward supporting the conventional construction process, more research into developing design for automation is essential. Our analysis shows that future research needs to seek alternatives in terms of construction material and engineering design that can support advanced automation and robotization in construction. While the review discussed a range of robotic technologies for different on-site building elements, more research is needed on how to streamline these often-complex systems into more simplified versions to develop a complete on-site robotic system for the construction of specific type of buildings. For instance, it could be studied how the 3D printing technology can be simplified and collaborated with climbing and wire robotics to construct high rise buildings. Above all, research that investigates what legal and operational standards and regulations are needed in support of widespread use of robotic technology is pressing to expand the awareness and advancement of automated methods in construction.

# Acknowledgements

This systematic review was completed as part of the first author’s PhD research and is funded by the Dean’s Discretionary Doctoral Scholarship, University of Auckland

# Appendix 1

Articles included in the systematic review.

- Concrete structures

| Author | Year | Country | Source | Robot Type | Robotic System Design/ Programme | Building Category |
| --- | --- | --- | --- | --- | --- | --- |
| Allwright, M. [42] | 2014 | Germany | Others | swarm robotics construction system | Self-Assembly | Low Rise |
| Balaguer, C. [43] | 2002 | Spain | IEEE | Automated Building Construction System | Controlled assembly | Low Rise |
| Bazhanov, A. [44] | 2016 | Russian Federation | IEEE | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Bos, F.P. [45] | 2017 | Netherlands | Others | 3D Printers | Rapid Prototyping | Low Rise |
| Bosscher, P. [46] | 2007 | USA | Automation in Construction | Wire robots for automated construction | Cartesian Motions | Low Rise |
| Buswell, R.A. [47] | 2007 | UK | Automation in Construction | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Cesaretti, G. [21] | 2014 | Italy | Others | 3D Printers | Rapid Prototyping | Low Rise |
| Davtalab, O. [48] | 2018 | USA | Automation in Construction | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Duballet, R. [49] | 2017 | France | Automation in Construction | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Ghaffar, S.H. [50] | 2018 | UK | Automation in Construction | 3D Printers | Rapid Prototyping | Low Rise |
| Gosselin, C. [51] | 2016 | France | Others | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Hack, N. [52] | 2014 | Switzerland | Architectural Design | Robotics for Moulds | Generic, Versatile Mobile robotics system | Low to Medium Rise |
| Heinrich, M. K. [53] | 2016 | Germany | IEEE | swarm robotics construction system | Self-organized construction | Medium to High Rise |
| Howe, A.S. [15] | 2000 | USA | Others | Automated Building Construction System | Controlled assembly | Low Rise |
| Howe, A.S. [54] | 2013 | USA | Others | Automated Building Construction System | Self-organized construction | Low Rise |
| Howe, A.S. [55] | 2015 | USA | Others | Automated Building Construction System | Self-organized construction | Low Rise |
| Hung, P.T. [56] | 2016 | Singapore | Others | 3D Printers | Rapid Prototyping | Low Rise |
| Hwang, D. [30] | 2005 | USA | ISARC | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Izard, Jean-Baptiste [57] | 2018 | France | Mechanisms and Machine Science | Wire robots for automated construction | Cartesian Motions | Low Rise |
| Teizer, J. [58] | 2016 | Germany | ISARC | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Khoshnevis, B. [37] | 2004 | USA | Automation in Construction | 3D Printers | Rapid Prototyping | Low Rise |
| Khoshnevis, B. [18] | 2006 | USA | Others | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Kim, H.H. [59] | 2017 | South Korea | IEEE | 3D Printers | Rapid Prototyping | Low Rise |
| Lim, S. [60] | 2011 | UK | ISARC | 3D Printers | Rapid Prototyping | Low Rise |
| Lublasser, E. [61] | 2018 | Germany | Automation in Construction | Robotics for Insulated Walls | Rapid Prototyping | Low Rise |
| Magnenat, S. [7] | 2012 | Switzerland | Autonomous Robots | swarm robotics construction system | Self-Assembly | Low to Medium Rise |
| Martinez, S. [62] | 2008 | Spain | Others | Automated Building Construction System | Controlled assembly | Low Rise |
| Meng, Y. [63] | 2008 | USA | IEEE | swarm robotics construction system | Self-Assembly | Low Rise |
| Moreira, E. [64] | 2015 | Portugal | IEEE | Wire robots for automated construction | Cartesian Motions | Low to Medium Rise |
| Nan, C. [65] | 2015 | Germany | Others | 3D Printers | Rapid Prototyping | Low Rise |
| Nematollahi, B. [32][66] | 2017 | Australia | ISARC | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Neudecker, S. [66] | 2016 | Germany | Others | Spraying Robotics | Rapid Prototyping | Low to Medium Rise |
| Kumar, Nitish [35] | 2017 | Switzerland | IEEE | Robotics for Moulds | Generic, Versatile Mobile robotics system | Low to Medium Rise |
| Panda, B. [34] | 2017 | Singapore | ISARC | 3D Printers | Rapid Prototyping | Low Rise |
| Panda, B. [67] | 2018 | Singapore | Others | 3D Printers | Rapid Prototyping | Low Rise |
| Sobotka, A. [68] | 2016 | Poland | Others | 3D Printers | Rapid Prototyping | Low to Medium Rise |
| Sobotka, A. [69] | 2017 | Poland | IOP | 3D Printers | Rapid Prototyping | Low Rise |
| Stroupe, A. [70] | 2005 | USA | IEEE | Robotic Construction Crew | Multi-robot Construction and Assembly | Low Rise |
| Vukorep, I. [71] | 2017 | Germany | ISARC | Wire robots for automated construction | Cartesian Motions | Low to Medium Rise |
| Werfel, J. [22] | 2005 | USA | Others | swarm robotics construction system | Self-Assembly | Low Rise |
| Werfel, J. [72] | 2006 | USA | IEEE | swarm robotics construction system | Self-Assembly | Low Rise |
| Werfel, J. [72] | 2014 | USA | Others | swarm robotics construction system | Self-Assembly | Low to Medium Rise |
| Wilkinson, S. [74] | 2016 | USA | Others | swarm robotics construction system | Self-organized construction | Low Rise |
| Williams II, R.L. [75] | 2008 | USA | ASME | Wire robots for automated construction | Cartesian Motions | Low Rise |
| Wismer, S. [76] | 2012 | Switzerland | IROS | Mobile Robotics | Controlled assembly | Low Rise |
| Zedin, T. [77] | 2017 | France | ISARC | Drilling Robots | Prototyping | Low to High Rise |
| Zhang, J. [78] | 2013 | USA | Automation in Construction | 3D Printers | Rapid Prototyping | Low to Medium Rise |

- Steel structures

| Author | Year | Country | Source | Robot Type | Robotic System Design/ Programme | Building Category |
| --- | --- | --- | --- | --- | --- | --- |
| Chu, B. [28] | 2009 | South Korea | ICARA | Automated Building Construction System | Controlled assembly | High Rise |
| Chu, B. [41] | 2013 | South Korea | Automation in Construction | Automated Building Construction System | Controlled assembly | High Rise |
| Ikeda, Y. [16] | 2006 | Japan | ISARC | Automated Building Construction System | Controlled assembly | High Rise |
| Jung, K. [79] | 2007 | South Korea | Automation and Robotics in Construction | Automated Building Construction System | Controlled assembly | High Rise |
| Jung, K. [38] | 2008 | South Korea | ICSMA | Automated Building Construction System | Controlled assembly | High Rise |
| Galloway, Kevin C. [80] | 2010 | USA | IEEE | Climbing Robot | Self-Assembly | Medium to High Rise |
| Lee, S. [40] | 2008 | South Korea | ICCAS | Automated Building Construction System | Controlled assembly | High Rise |
| Lee, S –K. [81] | 2007 | South Korea | ICCAS | Automated Building Construction System | Controlled assembly | High Rise |
| Nam, H. [82] | 2007 | South Korea | ICCAS | Automated Building Construction System | Controlled assembly | High Rise |
| Sakamoto, S. [17] | 1994 | Japan | ISARC | Automated Building Construction System | Controlled assembly | High Rise |
| Saltaren, R. [83] | 2004 | Spain | Others | Climbing Robot | Self-Assembly | Medium to High Rise |
| Yun, Seung-Kook [84] | 2011 | USA | Others | Automated Building Construction System | Self-Assembly | High Rise |

- Finishes (external/ internal)

| Author | Year | Country | Source | Robot Type | Robotic System Design/ Programme | Building Category |
| --- | --- | --- | --- | --- | --- | --- |
| Bruckmann, T. [85] | 2018 | Germany | Others | Wire robots for automated construction | Cartesian Motions | Low to Medium Rise |
| Choi, HS [24] | 2005 | South Korea | Automation in Construction | Tile Robots | Controlled assembly | Low to Medium Rise |
| Feng, C. [86] | 2015 | USA | Automation in Construction | brickwork robot | Controlled assembly | Low to High Rise |
| Pritschow, G. [87] | 1994 | Germany | IEEE | brickwork robot | Controlled assembly | Low to Medium Rise |
| Gambao, E. [5] | 1997 | Spain | IEEE | brickwork robot | Controlled assembly | Low to High Rise |
| Gambao, E [19] | 2000 | Spain | Automation in Construction | brickwork robot | Controlled assembly | Low to High Rise |
| Gil, M.S. [88] | 2011 | South Korea | ISARC | Glazing Robot Technology | Controlled assembly | High Rise |
| Han, C.S. [89] | 2006 | South Korea | Others | Glazing Robot Technology | Controlled assembly | High Rise |
| Hatano, M. [90] | 1999 | Japan | ISARC | brickwork robot | Controlled assembly | Low to High Rise |
| Helm, V. [91] | 2012 | Switzerland | IEEE | brickwork robot | Controlled assembly | Low to High Rise |
| Jung, D.H. [92] | 2014 | South Korea | ICCAS | Humanoid robot | Self-Assembly | Low Rise |
| King, N. [4] | 2014 | USA | Automation in Construction | Tile Robots | Prototyping | Low Rise |
| Lee, S. [93] | 2007 | South Korea | Automation and Robotics in Construction | Glazing Robot Technology | Controlled assembly | High Rise |
| Lee, S. [94] | 2008 | South Korea | ISARC | Glazing Robot Technology | Controlled assembly | High Rise |
| Lee, S. [95] | 2013 | South Korea | ISARC | Glazing Robot Technology | Controlled assembly | High Rise |
| Lee, S. [29] | 2015 | South Korea | Automation and Robotics in Construction and Mining: | Glazing Robot Technology | Controlled assembly | High Rise |
| Lee, S.Y. [27] | 2007 | South Korea | Autonomous Robots | Glazing Robot Technology | Controlled assembly | High Rise |
| Mattern, H. [96] | 2017 | Germany | Others | Wire robots for automated construction | Cartesian Motions | Low to Medium Rise |
| P. Gonzalez De Santos [97] | 2004 | Spain | IEEE | Mobile Robotics | Manually controlled robot | Low to High Rise |
| Schwartz, Mathew [98] | 2016 | South Korea | ACADIA | Humanoid Robot | Self-Assembly | Low Rise |
| Stumm, S. [99] | 2016 | Germany | Others | brickwork robot | Controlled assembly | Low to High Rise |
| Svoboda, P. [20] | 2011 | Czech Republic | ISARC | brickwork robot | Controlled assembly | Low to Medium Rise |
| Van Gassel, F. [26] | 2006 | Netherlands | ISARC | Glazing Robot Technology | Controlled assembly | High Rise |
| Vujovic, M. [99] | 2017 | Serbia | Others | Automated Building Construction System | Controlled assembly | Low Rise |
| Yu, S.N. [101] | 2005 | South Korea | ISARC | Glazing Robot Technology | Controlled assembly | High Rise |
| Yu, S.N. [25] | 2007 | South Korea | Autonomous Robots | Glazing Robot Technology | Controlled assembly | High Rise |
| Yulong Wu [102] | 2018 | Hong Kong | IEEE | Wire robots for automated construction | Cartesian Motions | Low to Medium Rise |

# References

- Munoz-Morera J, Maza I, Fernandez-Aguera CJ, Caballero F, Ollero A. Assembly planning for the construction of structures with multiple UAS equipped with robotic arms. 2015 International Conference on Unmanned Aircraft Systems (Icuas'15). 2015:1049-58.

- Prepare. with Modular Construction.

- Shaping the Future of Construction: A Breakthrough in Mindset and Technology. World Economic Forum; 2016 May 4,.

- King N, Bechthold M, Kane A, Michalatos P. Robotic tile placement: Tools, techniques and feasibility. Autom Constr. 2014;39:161-6.

- Gambao E, Balaguer C, Barrientos A, Saltaren R, Puente EA. Robot assembly system for the construction process automation. Part 3 (of 4), April 20, 1997 - April 25; 1997; Albuquerque, NM, USA: IEEE; 1997.

- Howe S. Designing for automated construction. Fourteenth International Symposium on Automation and Robotics in Construction; 05; Netherlands: Elsevier; 2000.

- Magnenat S, Philippsen R, Mondada F. Autonomous construction using scarce resources in unknown environments. Auton Robot. 2012 NOV;33(4):467-85.

- Oates BJ(, Capper G(. Using systematic reviews and evidence-based software engineering with masters students. BCS; 2009.

- Cooper H. The Integrative Research Review: A Systematic Approach Sage Publications: Beverly Hills, 1984, 143 pp. Educational Researcher. 1986;15(8):17-8.

- Dyba T, Kitchenham BA, Jorgensen M. Evidence-based software engineering for practitioners. S-M. 2005 Jan;22(1):58-65.

- Moher D, Shamseer L, Clarke M, Ghersi D, Liberati A, Petticrew M, et al. Preferred reporting items for systematic review and meta-analysis protocols (PRISMA-P) 2015 statement. Systematic reviews. 2015 Jan 1,;4(1):1.

- 1De Brún C, Pearce‐Smith N. Critical Appraisal. In: Searching Skills Toolkit. Oxford, UK: Wiley‐Blackwell; 2009. p. 103-4.

- Noblit GW. Meta-ethnography : synthesizing qualitative studies. Hare RD, editor. Newbury Park; Newbury Park, Calif.; London: Newbury Park : Sage Publications c1988; 1988.

- Using meta ethnography to synthesise qualitative research: a worked example. Journal of Health Services Research and Policy. 2002;7(4):209-15.

- Howe AS, Howe JW. Applying construction automation research to extraterrestrial building projects. 30th International Conference on Environmental Systems, July 10, 2000 - July 13; 2000; Toulouse, France: SAE International; 2000.

- Ikeda Y, Harada T. Application of the automated building construction system using the conventional construction method together. 2006 Proceedings of the 23rd International Symposium on Robotics and Automation in Construction, ISARC 2006; ; 2006.

- Sakamoto S, Mitsuoka H. Totally mechanized construction system for high-rise buildings (T-UP System). Proceedings of the 11th International Symposium on Automation and Robotics in Construction (ISARC); 24-26 May 1994; Amsterdam, Netherlands: Elsevier; 1994.

- Khoshnevis B, Hwang D, Yao K-, Yeh Z. Mega-scale fabrication by Contour Crafting. Int J Ind Syst Eng. 2006;1(3):301-20.

- Gambao E, Balaguer C, Gebhart F. Robot assembly system for computer-integrated construction. Autom Constr. 2000 SEP;9(5-6):479-87.

- Svoboda P, Usmanov V. Concept of a wall building industrial robotic system. Proceedings of the 28th International Symposium on Automation and Robotics in Construction, ISARC 2011; ; 2011.

- Cesaretti G, Dini E, De Kestelier X, Colla V, Pambaguian L. Building components for an outpost on the Lunar soil by means of a novel 3D printing technology. Acta Astronaut. 2014;93:430-50.

- Werfel J, Bar-Yam Y, Nagpal R. Building patterned structures with robot swarms. IJCAI International Joint Conference on Artificial Intelligence; ; 2005.

- Wu Y, Cheng HH, Fingrut A, Crolla K, Yam Y, Lau D. CU-brick cable-driven robot for automated construction of complex brick structures: From simulation to hardware realisation. IEEE; May 2018.

- Choi HS, Han CS, Lee KY, Lee SH. Development of hybrid robot for construction works with pneumatic actuator. Autom Constr. 2005 AUG;14(4):452-9.

- Yu SN, Lee SY, Han CS, Lee KY, Lee SH. Development of the curtain wall installation robot: Performance and efficiency tests at a construction site. Auton Robots. 2007;22(3):281-91.

- Van Gassel F, Schrijver P. Assembling wall panels with robotic technologies. 2006 Proceedings of the 23rd International Symposium on Robotics and Automation in Construction, ISARC 2006; ; 2006.

- Lee SY, Lee KY, Lee SH, Kim JW, Han CS. Human-robot cooperation control for installing heavy construction materials. Auton Robots. 2007;22(3):305-19.

- Chu B, Jung K, Chu Y, Hong D, Lim M-, Park S, et al. Robotic automation system for steel beam assembly in building construction. ICARA 2009 - Proceedings of the 4th International Conference on Autonomous Robots and Agents; ; 2009.

- Lee S, Moon JI. Case studies on glazing robot technology on construction sites. 32nd International Symposium on Automation and Robotics in Construction and Mining: Connected to the Future, Proceedings; ; 2015.

- Hwang D, Khoshnevis B. An innovative construction process-contour crafting (CC). 22nd International Symposium on Automation and Robotics in Construction, ISARC 2005; ; 2005.

- Castro-Lacouture D, Maynard C, Bryson LS, Williams II RL, Bosscher P. Concrete paving productivity improvement using a multi-task autonomous robot. Automation and Robotics in Construction - Proceedings of the 24th International Symposium on Automation and Robotics in Construction; ; 2007.

- Nematollahi B, Xia M, Sanjayan J. Current progress of 3D concrete printing technologies. ISARC 2017 - Proceedings of the 34th International Symposium on Automation and Robotics in Construction; ; 2017.

- Willmann J, Knauss M, Bonwetsch T, Apolinarska AA, Gramazio F, Kohler M. Robotic timber construction - Expanding additive fabrication to new dimensions. Autom Constr. 2016;61:16-23.

- Panda B, Lim JH, Mohamed NAN, Paul SC, Tay YWD, Tan MJ. Automation of robotic concrete printing using feedback control system. ISARC 2017 - Proceedings of the 34th International Symposium on Automation and Robotics in Construction; ; 2017.

- Kumar N, Hack N, Doerfler K, Walzer AN, Rey GJ, Gramazio F, et al. Design, development and experimental assessment of a robotic end-effector for non-standard concrete applications. IEEE; May 2017.

- Jung K, Chu B, Bae K, Lee Y, Hong D, Park S, et al. Development of automation system for steel construction based on robotic crane. ICSMA 2008 - International Conference on Smart Manufacturing Application; ; 2008.

- Khoshnevis B. Automated construction by contour crafting - Related robotics and information technologies. Autom Constr. 2004;13(1):5-19.

- Stuart-Smith R. Behavioural production: Autonomous swarm-constructed architecture. Archit Des. 2016;86(2):54-9.

- Camacho DD, Clayton P, O'Brien W, Ferron R, Juenger M, Salamone S, et al. Applications of additive manufacturing in the construction industry - A prospective review. ISARC 2017 - Proceedings of the 34th International Symposium on Automation and Robotics in Construction; ; 2017.

- Lee S, Nam H, Lee Y, Park J. A new type of bolting robot for steel-frame structure constructions. 2008 International Conference on Control, Automation and Systems, ICCAS 2008; ; 2008.

- Chu B, Jung K, Lim M-, Hong D. Robot-based construction automation: An application to steel beam assembly (Part I). Autom Constr. 2013;32:46-61.

- Allwright M, Bhalla N, El-Faham H, Antoun A, Pinciroli C, Dorigo M. Srocs: Leveraging stigmergy on a multi-robot construction platform for unknown environments. 9th International Conference on Swarm Intelligence, ANTS 2014, September 10, 2014 - September 12; 2014; Brussels, Belgium: Springer Verlag; 2014.

- Balaguer C, Abderrahim M, Navarro JM, Boudjabeur S, Aromaa P, Kahkonen K, et al. FutureHome: An integrated construction automation approach. IEEE Rob Autom Mag. 2002;9(1):55-66.

- Bazhanov A, Yudin D, Porkhalo V, Karikov E. Control system of robotic complex for constructions and buildings printing. IDT 2016 - Proceedings of the International Conference on Information and Digital Technologies 2016; ; 2016.

- Bos FP, Ahmed ZY, Wolfs RJM, Salet TAM. 3D printing concrete with reinforcement. High Tech Concrete: Where Technology and Engineering Meet - Proceedings of the 2017 fib Symposium; ; 2017.

- Bosscher P, Williams II RL, Bryson LS, Castro-Lacouture D. Cable-suspended robotic contour crafting system. Autom Constr. 2007;17(1):45-55.

- Buswell RA, Soar RC, Gibb AGF, Thorpe A. Freeform Construction: Mega-scale Rapid Manufacturing for construction. Autom Constr. 2007;16(2):224-31.

- Davtalab O, Kazemian A, Khoshnevis B. Perspectives on a BIM-integrated software platform for robotic construction through Contour Crafting. Autom Constr. 2018;89:13-23.

- Duballet R, Baverel O, Dirrenberger J. Classification of building systems for concrete 3D printing. Autom Constr. 2017;83:247-58.

- Ghaffar SH, Corker J, Fan M. Additive manufacturing technology and its implementation in construction as an eco-innovative solution. Autom Constr. 2018;93:1-11.

- Gosselin C, Duballet R, Roux P, Gaudillière N, Dirrenberger J, Morel P. Large-scale 3D printing of ultra-high performance concrete - a new processing route for architects and builders. Mater Des. 2016;100:102-9.

- Hack N, Lauer WV. Mesh-mould: Robotically fabricated spatial meshes as reinforced concrete formwork. Archit Des. 2014;84(3):44-53.

- Heinrich MK, Wahby M, Soorati MD, Hofstadler DN, Zahadat P, Ayres P, et al. Self-Organized Construction with Continuous Building Material: Higher Flexibility Based on Braided Structures. IEEE; Sep 2016.

- Howe AS, Wilcox B, McQuin C, Townsend J, Rieber R, Barmatz M, et al. Faxing Structures to the Moon: Freeform Additive Construction System (FACS). AIAA SPACE 2013 Conference and Exposition; 10-12 Sept. 2013; Reston, VA, USA: American Institute of Aeronautics and Astronautics, Inc; 2013.

- Howe AS, Wilcox B, McQuin C, Mittman D, Townsend J, Polit-Casillas R, et al. Modular Additive Construction Using Native Materials. 14th Biennial International Conference on Engineering, Science, Construction and Operations in Challenging Environments; 27-29 Oct. 2014; Reston, VA, USA: American Society of Civil Engineers; 2015.

- Hung PT, Hui LJ, Pham Q-. Robotic 3D-printing for building and construction. Proceedings of the International Conference on Progress in Additive Manufacturing; ; 2016.

- Izard J, Dubor A, Herve P, Cabay E, Culla D, Rodriguez M, et al. On the improvements of a cable-driven parallel robot for achieving additive manufacturing for construction. 3rd International Conference on Cable-Driven Parallel Robots, CableCon 2017, August 2, 2017 - August 4; 2017; Quebec City, Canada: Springer Netherlands; 2018.

- Teizer J, Blickle A, King T, Leitzbach O, Guenther D. Large scale 3D printing of complex geometric shapes in construction. ISARC 2016 - 33rd International Symposium on Automation and Robotics in Construction; ; 2016.

- Kim HH, Park CY, Lee MC. A study on the 3D printing simulator for construction and application of robust control Using SMCSPO. IEEE; 2017.

- Lim S, Buswell R, Le T, Wackrow R, Austin S, Gibb A, et al. Development of a viable concrete printing process. Proceedings of the 28th International Symposium on Automation and Robotics in Construction, ISARC 2011; ; 2011.

- Lublasser E, Adams T, Vollpracht A, Brell-Cokcan S. Robotic application of foam concrete onto bare wall elements - Analysis, concept and robotic experiments. Autom Constr. 2018;89:299-306.

- Martinez S, Jardon A, Maria JM, Gonzalez P. Building industrialization: Robotized assembly of modular products. Assem Autom. 2008;28(2):134-42.

- Meng Y, Gan J. A distributed swarm intelligence based algorithm for a cooperative multi-robot construction task. 2008 IEEE Swarm Intelligence Symposium, SIS 2008; ; 2008.

- Moreira E, Pinto AM, Costa P, Moreira AP, Veiga G, Lima J, et al. Cable robot for non-standard architecture and construction: A dynamic positioning system. Proceedings of the IEEE International Conference on Industrial Technology; ; 2015.

- Nan C. A new machinecraft: A Critical evaluation of architectural robots [Internet]; 2015 [cited 18 June 2018]. Available from: https://www.scopus.com/inward/record.uri?eid=2-s2.0-84983559926&doi=10.1007%2f978-3-662-47386-3_23&partnerID=40&md5=59bccd2d73cb1018abf5c941600e0d91.

- Neudecker S, Bruns C, Gerbers R, Heyn J, Dietrich F, Dr�der K, et al. A New Robotic Spray Technology for Generative Manufacturing of Complex Concrete Structures Without Formwork. Procedia CIRP; ; 2016.

- Panda B, Paul SC, Hui LJ, Tay YWD, Tan MJ. Additive manufacturing of geopolymer for sustainable built environment. J Clean Prod. 2018;167:281-8.

- Sobotka A, Pacewicz K. Building Site Organization with 3D Technology in Use. Procedia Engineering; ; 2016.

- Sobotka A, Pacewicz K. Mechanisation and automation technologies development in work at construction sites. IOP conference series. 2017;251(1):012046.

- Stroupe A, Huntsberger T, Okon A, Aghazarian H, Robinson M. Behavior-based multi-robot collaboration for autonomous construction tasks. 2005 IEEE/RSJ International Conference on Intelligent Robots and Systems, IROS; ; 2005.

- Vukorep I. Autonomous big-scale additive manufacturing using cable-driven robots. ISARC 2017 - Proceedings of the 34th International Symposium on Automation and Robotics in Construction; ; 2017.

- Werfel J, Bar-Yam Y, Rus D, Nagpal R. Distributed construction by mobile robots with enhanced building blocks. Proceedings - IEEE International Conference on Robotics and Automation; ; 2006.

- Werfel J, Petersen K, Nagpal R. Designing collective behavior in a termite-inspired robot construction team. Science. 2014;343(6172):754-8.

- Wilkinson S, Musil J, Dierckx J, Gallou I, de Kestelier X. Autonomous Additive Construction on Mars. 15th Biennial International Conference on Engineering, Science, Construction and Operations in Challenging Environments; 11-15 April 2016; Reston, VA, USA: ASCE - American Society of Civil Engineers; 2016.

- Williams II RL, Xin M, Bosscher P. Contour-crafting-cartesian-cable robot system concepts: Workspace and stiffness comparisons. Proceedings of the ASME Design Engineering Technical Conference; ; 2008.

- Wismer S, Hitz G, Bonani M, Gribovskiy A, Magnenat S. Autonomous construction of a roofed structure: Synthesizing planning and stigmergy on a mobile robot. 2012 IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS 2012); 7-12 Oct. 2012; Piscataway, NJ, USA: IEEE; 2012.

- A method based on C-K Theory for fast STCR development: The case of a drilling robot design. ISARC 2017 - Proceedings of the 34th International Symposium on Automation and Robotics in Construction. 2017:464-71.

- Zhang J, Khoshnevis B. Optimal machine operation planning for construction by Contour Crafting. Autom Constr. 2013;29:50-67.

- Jung K, Kim D, Bae K, Hong D. Developement of the gripping control algorithm for wire-suspended object in steel construction. Automation and Robotics in Construction - Proceedings of the 24th International Symposium on Automation and Robotics in Construction; ; 2007.

- Galloway KC, Jois R, Yim M. Factory Floor: A Robotically Reconfigurable Construction Platform. 2010 Ieee International Conference on Robotics and Automation (Icra). 2010:2467-72.

- Lee S-, Doh NL, Park G-, Kang K-, Lim M-, Hong D-, et al. Robotic technologies for the automatic assemble of massive beams in high-rise building. ICCAS 2007 - International Conference on Control, Automation and Systems; ; 2007.

- Nam H, Choi W, Ryu D, Lee Y, Lee S-, Ryu B. Design of a bolting robot for constructing steel structure. ICCAS 2007 - International Conference on Control, Automation and Systems; ; 2007.

- Saltaren R, Aracil R, Reinoso O. Analysis of a climbing parallel robot for construction applications. Computer-Aided Civil and Infrastructure Engineering. 2004;19(6):436-45.

- Yun S, Schwager M, Rus D. Coordinating construction of truss structures using distributed equal-mass partitioning. Robotics Research - The 14th International Symposium ISRR; Springer Verlag; 2011.

- Bruckmann T, Reichert C, Meik M, Lemmen P, Spengler A, Mattern H, et al. Concept studies of automated construction using cable-driven parallel robots [Internet]; 2018 [cited 19 June 2018]. Available from: https://www.scopus.com/inward/record.uri?eid=2-s2.0-85025840699&doi=10.1007%2f978-3-319-61431-1_31&partnerID=40&md5=785a51c4941dc425257db0d606c02655.

- Feng C, Xiao Y, Willette A, McGee W, Kamat VR. Vision guided autonomous robotic assembly and as-built scanning on unstructured construction sites. Autom Constr. 2015;59:128-38.

- Pritschow G, Dalacker M, Kurz J, Zeiher J. Mobile robot for on-site construction of masonry. IEEE/RSJ/GI International Conference on Intelligent Robots and Systems; ; 1994.

- Gil MS, Kang MS, Lee SH, Kim D, Kim B-, Han CS. Proposal of installation method of heavy duty glass using intuitive manipulation device. Proceedings of the 28th International Symposium on Automation and Robotics in Construction, ISARC 2011; ; 2011.

- Han CS, Lee SY, Lee KY, Park BS. A multidegree-of-freedom manipulator for curtain-wall installation. J Field Rob. 2006;23(5):347-60.

- Hatano M, Minami M, Asakura T, Ohsumi T. Construction robot systems using mobile manipulators-piling up blocks with real systems and positioning accuracy. Proceedings of 16th International Symposium on Robotics and Automation in Construction (ISARC 99); 22-24 Sept. 1999; Madrid, Spain: Univ. Carlos III de Madrid; 1999.

- Helm V, Ercan S, Gramazio F, Kohler M. Mobile robotic fabrication on construction sites: DimRob. IEEE International Conference on Intelligent Robots and Systems; ; 2012.

- Jung DH, Park J, Schwartz M. Towards on-site autonomous robotic floor tiling of mosaics. International Conference on Control, Automation and Systems; ; 2014.

- Lee S, Lee K, Gil M, Lee S, Han C. Design of a ceiling glass installation robot. Automation and Robotics in Construction - Proceedings of the 24th International Symposium on Automation and Robotics in Construction; ; 2007.

- Lee S, Lee J, Han C, Lee K, Lee S. Human robot cooperative control and task planning for a glass ceiling installation robot. ISARC 2008 - Proceedings from the 25th International Symposium on Automation and Robotics in Construction; ; 2008.

- Lee S, Eom S, Moon J. Design of teach pendant for robot glazing system. ISARC 2013 - 30th International Symposium on Automation and Robotics in Construction and Mining, Held in Conjunction with the 23rd World Mining Congress; ; 2013.

- Mattern H, Bruckmann T, Spengler A, K�nig M. Simulation of automated construction using wire robots. Proceedings - Winter Simulation Conference; ; 2017.

- P. Gonzalez De Santos, J. Estremera, E. Garcia, M. Armada. A service robot for construction industry. Proceedings World Automation Congress, 2004. ; 2004.

- Schwartz M. Use of a low-cost humanoid for tiling as a study in on-site fabrication. 36th Annual Conference of the Association for Computer Aided Design in Architecture - Posthuman Frontiers: Data, Designers, and Cognitive Machines, ACADIA 2016, October 27, 2016 - October 29; 2016; Ann Arbor, MI, United states: ACADIA; 2016.

- Stumm S, Braumann J, Brell-Cokcan S. Human-Machine Interaction for Intuitive Programming of Assembly Tasks in Construction. Procedia CIRP. 2016;44:269-74.

- Vujovic M, Rodic A, Stevanovic I. Design of modular re-configurable robotic system for construction and digital fabrication [Internet]; 2017 [cited 19 June 2018]. Available from: https://www.scopus.com/inward/record.uri?eid=2-s2.0-85007285946&doi=10.1007%2f978-3-319-49058-8_60&partnerID=40&md5=d4d8378862cdf3acd4e4ccea4fb889c7.

- The analysis of the curtain wall installation robot: Based on the test in the construction site. 2005.

- Yulong Wu, Hung Hon Cheng, Adam Fingrut, Kristof Crolla, Yeung Yam, Darwin Lau. CU-brick cable-driven robot for automated construction of complex brick structures: From simulation to hardware realisation. Piscataway: The Institute of Electrical and Electronics Engineers, Inc. (IEEE); Jan 1, 2018.